# Smart Measuring Device

## Aim

To measure the distance between the device and an object and display the measurement on an OLED screen.

## What is the project?

A digital distance-measuring device that uses an ultrasonic sensor to estimate distance without physically touching an object.

## How does it work?

The ultrasonic sensor emits a sound pulse above the range of human hearing.

The pulse reflects off an object and returns to the sensor.

The ESP32 measures the round-trip travel time.

Distance is calculated as speed of sound multiplied by travel time, divided by two.

The calculated distance is displayed on the OLED screen, in inches, feet and centimeters.

The laser light can act as a visual pointer for the measuring direction, depending on its placement.

The push button can freeze the measurement on the screen, so that it can be looked at.

## Components used

ESP32 microcontroller

Ultrasonic sensor

OLED display

Laser light module

Breadboard

Jumper wires

Push Button

## Connections

| Component / Pin | Typical connection |
| --- | --- |
| ESP32 5V | Breadboard positive |
| ESP32 GND | Breadboard negative |
| Ultrasonic VCC | Breadboard positive |
| Ultrasonic GND | Breadboard negative |
| Ultrasonic TRIG | Pin 5 (ESP) |
| Ultrasonic ECHO | Pin 18 (ESP) |
| OLED VCC OLED GND | Breadboard positive Breadboard negative |
| OLED SDA OLED SCL/SCK | Pin 21 (ESP) Pin 22 (ESP) |
| Laser module S Laser module + Laser module - Push Button Push Button | Pin 15 (ESP) Breadboard positive Breadboard negative Pin 4 (ESP) Breadboard negative |